INTEQ gyro models

Inputs to the “ISCWSA” format model, per SPE 90408

1.	Seeker error model

Error term group	Class	Used	Parameters/Definitions

XY Accelerometer	GEN	Yes	cant angle = 0, Switching = No

XY Stationary Gyro	STAT	Yes	start_inc = 0, end_inc = 80, init_inc =

Tool Misalignment	GEN	Yes	Alt. 3

Depth			GEN	Yes

XY Accelerometer (GEN)

Error term				Mode	Value

xy accelerometer bias			R	.01 m/s2

Accelerometer scale factor error	S	.0005

Accelerometer misalignment		S	0

Gravity bias				S	.004 m/s2

XY Stationary Gyro (STAT)

Error term				Mode	Value

xy gyro bias 1				S	0

xy gyro bias 2				S	0

xy gyro random noise			R	.02 deg/hr

xy gyro g-dept error 1			S	.07 deg/hr/g

xy gyro g-dept error 2			S	0

xy gyro g-dept error 3			S	0

xy gyro g-dept error 4			S	.07 deg/hr/g

Gyro scale factor error		S	.002

Gyro misalignment			S	0

Tool Misalignment (GEN)

Error term				Mode	Value

xy Misalignment 1			S	.05 deg

xy Misalignment 2			S	.05 deg

xy Misalignment 3			S	.1 deg

xy Misalignment 4			S	.1 deg

Vertical Sag				S	.05 deg

Depth (GEN)

Error term				Mode	Value

Depth Random Error			R	0

Depth Systematic Reference		S	.2 m

Depth Scale				S	1.5.E-3

Depth Stretch-type			G	2.E-7 m-1

1.	Seeker error model (corrections and applications)

Available corrections : 			Effect

DLS						Removes the DLS term  (see Note 1)

Available applications  (see Note 2):

Application	Situation			Effect

Drillpipe	Tool run inside drillpipe	xy Misalignment terms are increased by 50%

Bent sub	Run above bent sub		Vertical Sag = 0,

all xy Misalignments set to 0.5 deg

Single shot	Depth tied to drillpipe		Depth random error = 0.35 m,

Depth systematic reference = 0,

Depth scale = 6.0E-4 propagating globally, Depth stretch-type = 2.2E-7 m-1

Floater	Tool run on floating rig	Depth random error = 2.2 m,

Depth systematic reference = 1 m

2.	RIGS error model

Error term group		Class	Used	Parameters/Definitions

XYZ Accelerometer		GEN	Yes

XYZ Stationary Gyro	STAT	Yes	start_inc = 0, end_inc = 0, init_inc = -1

XYZ Continuous Gyro	CONT	Yes	start_inc = 0, end_inc = 180,

running_speed = 1 m/s

Tool Misalignment		GEN	Yes	Alt. 3

Depth				GEN	Yes

Minimum distance between 		Noise reduction factor at

initializations = 9999 m			initialization = 1.0

XYZ Accelerometer (GEN)

Error term				Mode	Value

xy accelerometer bias			S	 0.002 m/s2

z accelerometer bias			S	 0.002 m/s2

Accelerometer scale factor error	S	0.0002

Accelerometer misalignment		S	0.002 deg

XYZ Stationary Gyro (STAT)

Error term				Mode	Value

xy gyro bias 1				S	0.0035 deg/hr

xy gyro bias 2				S	0.0035 deg/hr

xy gyro random noise			R	0.0086 deg/hr

xy gyro g-dept error 1			S	0

xy gyro g-dept error 2			S	0

xy gyro g-dept error 3			S	0

xy gyro g-dept error 4			S	0

z gyro bias 				S	0.0035 deg/hr

z gyro random noise			R	0.0086 deg/hr

z gyro g-dept error 1			S	0

z gyro g-dept error 2			S	0

Gyro scale factor error		S	1.E-5

Gyro misalignment			S	0.021 deg

XYZ Continuous Gyro (CONT)

Error term				Mode	Value

xyz Gyro drift				S	0.01 deg/hr

xyz Gyro random walk		S	0.01 deg/√hr

2.	RIGS error model (continued)

Tool Misalignment (GEN)

Error term				Mode	Value

xy Misalignment 1			S	0.0125 deg

xy Misalignment 2			S	0.0125 deg

xy Misalignment 3			S	0.025 deg

xy Misalignment 4			S	0.025 deg

Vertical Sag				S	0.05 deg

Depth (GEN)

Error term				Mode	Value

Depth Random Error			R	0

Depth Systematic Reference		S	0.1m

Depth Scale				S	2.5E-4

Depth Stretch-type			S	7.E-8 m-1

Available applications :

Application	Situation			Effect

Floater	Tool run on floating rig	Depth random error = 2.2 m,

Depth systematic reference = 1 m

3.	FINDS error model

Error term group		Class	Used	Parameters/Definitions

Tool Misalignment		GEN	Yes	Alt. 3

Depth				GEN	Yes

Tool Misalignment (GEN)

Error term				Mode	Value

xy Misalignment 1			S	0.029 deg

xy Misalignment 2			S	0.029 deg

xy Misalignment 3			S	0.029 deg

xy Misalignment 4			S	0.029 deg

Vertical Sag				S	0

Depth (GEN)

Error term				Mode	Value

Depth Random Error			R	0

Depth Systematic Reference		S	0

Depth Scale				S	5.0E-4

Depth Stretch-type			S	0